# PCB:

136x180mm
Fr4 double layer 
2oz, 8mil clearance

# Case (zs-8247)

(optional: color of the case: black)

Vendor Ningbo Sunyoung Enclosure Co.,Ltd 
Address: Room802,TianHong Building ,No 555 ,RenheRoad ,Cultural Business Zone,Cixi  ,Ningbo China 315300
Tel: +86-574-63086395  Fax : ++86-574-63086395
Mobile :+8615157753600   
Skype :+8615157753600  Whats App :+ 8615157753600
Wechat : cjh15157753600

E-mail : sales2@robustcasing.com
www.robustcasing.com
https://robustcase.en.alibaba.com/

# Plate drill/mill guid

Front Platte

Backe Platte

# Engraving/Labeling

Front

Back

IEC C14
Male

IEC C13
Female

Assembly

Panel Front:

On the Front Panel two IEC connectors have to be added. Left one is the male C14 and the left connector is Female C13.

Panel Back:

4 Fuse holder (20x5mm) have to be mounted on the BackPanel

!ALL cabels must have a shrinking tube

When the pcb got assembled, a few wires must be added.

P1, P2 -> 2.5mm^2  (P1 needs to be connected to IEC C14 and P2 needs to be connected to IEC C13) 
!ALL cabels must have a shrinking tube

P10 -> 2.5mm^2 (to Fuse 1)

P9 -> 1.5mm^2

- 1 ->  Fuse2

- 2 -> Fuse3

- 3 -> Fuse2

- 4 -> Fuse3

- 5 -> Fuse 4

- 6 -> Fuse 4

!ALL cabels must have a shrinking tube

The pcb shall be placed as shown in the following picture

# Finish

And in the End the fuses shall be put in like:

- Fuse1, 230VAC   ~15A   AC

- Fuse2, 24VDC   ~4A   DC

- Fuse3, 24VDC   ~4A   DC

- Fuse4, 24VDC   ~10A   DC